EART33001 – Week 3 Practical: ICP-OES and ICP-MS

Q1) Calculate the frequency and energy (in eV) of green visible light ( = 550 nm).

Note the following:  cvac = 2.998 x 108 m s-1

h = 6.6262 x 10-34 J s

1 eV = 1.6022 x 10-19 J

c = 

E = h 

Frequency  = c/ = 2.998x108 / 550x10-9 = 5.451x1014 s-1

Energy E = h 6.6262 x 10-34 × 5.451x1014 = 3.61x10-19 J = 2.25 eV

Q2) Hydrogen emission frequencies in the visible region are given by the Balmer series, which is summarised by the following equation:

n=3.290 x 1015 14- 1b2;  in s-1 and with b=3,4,5,6,…

What are the wavelengths of the first four emission lines?

b = 3; n = 4.57 x 1014;  = c/n = 2.998x108 / 4.57 x 1014 = 656 nm

b = 4; n = 6.17 x 1014;  = c/n = 2.998x108 / 6.17 x 1014 = 486 nm

b = 5; n = 6.91 x 1014;  = c/n = 2.998x108 / 6.91 x 1014 = 434 nm

b = 6; n = 7.31 x 1014;  = c/n = 2.998x108 / 7.31 x 1014 = 410 nm

Q3) Find  and k for 2.50 eV photons. (k [wavenumber] = 1/

2.5 eV = 4.0055 x 10-19 J

Frequency  = E/h = 4.0055 x 10-19 / 6.6262 x 10-34 = 6.0449 x 1014 s-1

Wavelength  = c/ = 2.998 x 108 / 6.0449 x 1014 = 496 nm

Wavenumber k = 1/=2.02 x 106 m-1 = 2.02 x 10-3 nm-1

Q4) Find cH2O and H2O of sodium light (air = 5893 A) at 25oC.

The refractive index (nH2O) of water at this wavelength and temperature is 1.33.

Assume that the frequency is the same in both media. Recall that: nH2O=caircH2O

c(H2O) = c(air)/n(H2O) = 2.998 x 108 / 1.33 = 2.25 x 108 m s-1

Frequency (air) = c(air)/(air) = 2.998 x 108 / 5893 x 10-10 = 5.09 x 1014 s-1 ≈ (H2O)

(H2O) = c(H2O) / (H2O) = 2.25 x 108 / 5.09 x 1014 = 4.43 x 10-7 m = 443.1 nm

Q5) The track pitch (or groove spacing t) on a DVD is 0.74 m. Visible light covers the wavelength range between approximately 0.390 and 0.700 m. The diffraction grating equation is: n = t . sin , where n = order of diffraction.

At what range of angles would you detect diffraction maxima from visible light off a DVD surface (use an order of diffraction of 1)?

sin-1 (0.39/0.74) = 31.8° and sin-1 (0.70/0.74) = 71.1°

Q6) Below is a table of strong optical emission lines, masses of the different isotopes, and percent mass abundances for the different isotopes for 8 common metals in environmental systems.

Determine where there may be problems with spectral overlap for these elements in an ICP-OES instrument with a 10 A spectral resolution?

Similarly, determine where there may be isobaric interferences causing potential issues for ICP-MS analyses using a quadrupole ICP-MS with a 1 amu mass resolution.

ICP-OES – Possible interferences:

Mg at 2852 A and As at 2860 A

V at 3111 A and Fe at 3111 A

ICP-MS – Possible interference:

40K at 39.964 and 40Ca at 39.963

58Fe at 57.933 and 58Ni at 57.935

Q7) In the spreadsheet labelled “Fluid Analyses”, along with the data for the Rio Tanama problem, there are two sheets of data, one for ICP-OES analyses of calibration standards and an unknown, and one for ICP-MS analysis of calibration standards and a different unknown. Note that errors for these types of analyses are usually given as 3, that is, ± 3 x standard deviation.

a) Calculate calibration curves for ICP-OES and determine whether the linear relationship holds.

Plotting standard analyses as unknown shows linear calibration holds for the 3 elements.

b) Give the concentrations in the unknown analysed by ICP-OES with error bars.

c) Using the mean of 3 runs for each standards, calculate calibration curves for ICP-MS analyses of 29Si, 63Cu, 65Cu, 206Pb, and 208Pb and determine whether the linear relationship holds for each calibration. An acceptable calibration should have a correlation coefficient R2 > 0.95.

If there are species for which it does not hold, what could be the cause(s)?

Plotting standard analyses shows linear calibration holds for the 2 isotopes of Cu and Pb, as calibration lines have R2 > 0.99. Linear calibration does not hold for Si, indicated by low R2 of 0.92.

This is because the torch of the ICP-MS is made of silica, making ICP-MS analyses of Si imprecise. So we typically analyse Si using ICP-OES. In this case, the sensitivity of ICP-MS is both an advantage and a problem compared to ICP-OES.

d) Give the concentrations in the unknown analysed by ICP-MS with error bars.

e) There appear to be some complications concerning some of the ICP-MS data. Discuss those complications and give possible explanations for them.

Along with the Si analysis problem in the ICP-MS analysis discussed above (exemplified here with negative intensity for 29Si), there is also a discrepancy in the Pb abundance results. The isotope 206Pb gives 84 ppb, while analysing the isotope 208Pb gives 18 ppb. This is probably because the 206Pb/208Pb isotope ratio in the analysed sample does not match the known relative abundance of isotopes 206Pb and 208Pb in natural Pb.  Therefore, because 206Pb is a radiogenic isotope produced by radioactive decay of 238U, and since the Pb abundance calculated using 206Pb is higher than that calculated using 208Pb, this analysis suggests that the fluid has been in contact with elements that are part of the 238U decay chain. This is in fact the case.